Citrix WinView

Application Notes

Gandalf's LanLine ISDN Bridge Products

This application note is for

informational use only and

Citrix makes no representations

or warranties with respect to the

contents or use of this document

or of any of third-party products

discussed within.

September 1, 1994

Citrix Systems

210 University Drive

Suite 700

Coral Springs, FL 33071

(305) 755-0559

FAX (305) 341-6880

Overview:

---------

This application note facilitates the configuration of Gandalf's LanLine ISDN

Bridge Products, referred to as "Gandalf Bridge", with Citrix "WinView for Networks".

Gandalf Bridge products provideremote networking over ISDN lines.  LANLine remote

access products support both IP and IPX networks, providing a bridge between the "Local"

network and a "Remote client or network".  Remote PC's can connect to "Local" WinView

Application servers using the Citrix Lanlink client software.  In addition, on-Lan PC's on

the "Local" side can connect to "Remote" WinView Application servers using the Citrix

Lanlink client software.

Disclaimer:

-----------

The scenarios described in this document have been tested by Citrix.

Other variations to the scenarios described in this document may work, however

they have not specifically been tested by Citrix.  In order to recreate the

configurations, you should use the specified revision levels of all software

products described in this document and stay within the bounds of the features

and functions described in this document.

Please note that this application note is a living document and will be

modified as new information and versions of the software described herein

become available.  Make sure you have the latest version of this document

before you begin.  The latest version is always available in the Citrix Forum

on Compuserve.

Requirements:

-------------

Gandalf "LANLine" Series Intelligent bridge on both the remote and local side.

ISDN lines installed at both the local and remote site

Citrix "WinView for Networks" Application Server Software - minimum one copy

Setup:

------

1) It is suggested that you follow the individual steps in the

Gandalf User's Manual to install the LANLine Series bridges

2) Install the WinView Application Server software as per the

Citrix Installation Guide.

3) Configure a Citrix LanLink client as per the Citrix Lanlink

User's Manual

Operation:

----------

1) If the Gandalf Bridges are operating and the ISDN lines are active,

the "remote" network should appear as if it is local.  Depending on

Gandalf bridge equipment purchased your bandwidth will be either

128kbps or 384kbps "uncompressed".  Gandalf's data compression

combined with the Citrix Lanlink compression provide excellent

performance for several simultaneous connections over the bridge.

2) The LANLine bridges can be configured and managed remotely by

any SNMP management system.

3) From a PC Client run the Lanlink program.  From the LanLink main

menu, Choose "Connect to AppServer", highlight the desired Appserver

and press enter. Save the configuration,F4, From the Lanlink Main Menu

select "Connect to and Appser", highlight the desired selection and press

enter.  A WinView Login prompt should appear momentarily.

Notes:

------

The Gandalf LANLine Series Bridge products are an excellent way to access

remote LAN's.  Special features on the LANLine series Bridges allow connections

to be initiated or dropped, transparently to users, giving the remote office a

virtual presence on the central site LAN, while keeping "line charges at a minimum".

Combined with data compression rates of up to 8:1, LANLine solutions can make

ISDN an affordable alternative.

In addition, LanLink greatly improves the performance of the ISDN Bridge because the

execution of the applications is performed on the WinView server

only requiring screen updates and keystrokes to be sent across the

bridge.  The majority of the network traffic will be between the WinView

server and the Netware server which should be on the same local network.

Problems:

---------

There are no known problems when using Gandalf LANLine Series Bridge Products

in conjunction  with WinView for Networks Application Server software.

#