Robert Pitruzzella

7/10/96

Digi Retoura ISDN Router with WinFrame

Overview

The purpose of this document is to facilitate the  installation and configuration of the Digi Retoura ISDN Router  with WinFrame.   The Digi Retoura ISDN router product provides remote ethernet network  routing over standard basic rate 2B+D (2x 64Kpps data channels) ISDN lines. Ethernet_II frames are transmitted between  the local and remote networks over the ISDN line.   This makes the remote PC or LAN appear  as one contiguous network.  The Retoura routes the  TCP/IP protocol keeping local IP traffic on each side of the Retouras, sending only those packets which need to reach machines on the other segment. This greatly enhances the performance by not allowing bandwidth to be taken up by unnecessary traffic. The WinFrame client can establish a remote control session on a WinFrame server as if it were on the local LAN.

Requirements

Hardware Requirements:

1-Intel based computer with WinFrame server installed (network adapter required)

1-intel based computer with the WinFrame client installed (network adapter required )

2-Digi Retoura ISDN Routers (hardware revision R1480 or higher)

2-ISDN phone lines (provided from you local ISDN provider)

Software Requirements:

WinFrame server and client software version 1.6 or higher

(TCP/IP protocol installed on both machines)

Installation

- Install the Digi Retoura following the steps in the User Installation manual.

- Install and configure a WinFrame server on one side of the  Digi routers.  The WinFrame server should have TCP/IP networking installed.

- Once the bridge between the two networks is  established,  the client machine will be able to use ping.exe to ping  the WinFrame server on the other side to verify network connectivity.

- note: the ping.exe file is provided from your TCP/IP protocol software.

- Install and configure a WinFrame TCP/IP client connection on the client machine to connect to the WinFrame server on the other side of the Retoura.

- Start the WinFrame client on the client PC and connect to the WinFrame server on the other network.

Note:

Your ISDN provider  should supply you with the following information for configuring the Retoura:

SPID number(s) - (Service provider identification number)

Switch type

Area code and phone number